#### 16AISS16AO2

#### 16-Bit: 16 Ch Analog Input, 2 Ch Analog Output, #### 6-Bit Digital Input/Output

2K\XP\Vista\Win7 Driver User Manual

Manual Revision: June 6, 2011

General Standards Corporation 8302A Whitesburg Drive Huntsville, AL 35802 Phone: (256) 880-8787 Fax: (256) 880-8788 URL: http://www.generalstandards.com E-mail: sales@generalstandards.com E-mail: support@generalstandards.com

Preface

Copyright ©2002, General Standards Corporation

Additional copies of this manual or other literature may be obtained from:

General Standards Corporation

8302A Whitesburg Dr.

Huntsville, Alabama 35802

Phone: (256) 880-8787

FAX: (256) 880-8788

URL: http://www.generalstandards.com

E-mail: sales@generalstandards.com

General Standards Corporation makes no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. Although extensive editing and reviews are performed before release to ECO control, General Standards Corporation assumes no responsibility for any errors that may exist in this document. No commitment is made to update or keep current the information contained in this document.

General Standards Corporation does not assume any liability arising out of the application or use of any product or circuit described herein, nor is any license conveyed under any patent rights or any rights of others.

General Standards Corporation assumes no responsibility for any consequences resulting from omissions or errors in this manual or from the use of information contained herein.

General Standards Corporation reserves the right to make any changes, without notice, to this product to improve reliability, performance, function, or design.

ALL RIGHTS RESERVED.

The Purchaser of this software may use or modify in source form the subject software, but not to re-market or distribute it to outside agencies or separate internal company divisions. The software, however, may be embedded in the Purchaser’s distributed software. In the event the Purchaser’s customers require the software source code, then they would have to purchase their own copy of the software.

General Standards Corporation makes no warranty of any kind with regard to this software, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose and makes this software available solely on an “as-is” basis. General Standards Corporation reserves the right to make changes in this software without reservation and without notification to its users.

The information in this document is subject to change without notice. This document may be copied or reproduced provided it is in support of products from General Standards Corporation. For any other use, no part of this document may be copied or reproduced in any form or by any means without prior written consent of General Standards Corporation.

GSC is a trademark of General Standards Corporation.

PLX and PLX Technology are trademarks of PLX Technology, Inc.

Table of Contents

- Scope……………………………………………………………………………..…………..	  4

- Hardware Overview……………………………………………………………..………….	  5

- Referenced Documents…………………………………………………….……………..	  6

- General Standards API……………………………………………………….……………	  7

  - AISS16AO2_66_FindBoards()………………………………………………………...	  8

  - AISS16AO2_66_Get_Handle()…………………………………………………………	  9

  - AISS16AO2_66_Read_Local32()……………………………………………………...	10

  - AISS16AO2_66_Write_Local32()……………………………………………………...	11

  - AISS16AO2_66_Close_Handle()………………………………………………………	12

  - Interface Functions………………………………………………………….…………	13

    - AISS16AO2_66_Initialize()……………………………………………………	13

    - AISS16AO2_66_Autocal()…………………………………………………….	14

    - AISS16AO2_66_Set_Input _Mode()……………………..………………….	15

    - AISS16AO2_66_Set_Output _Mode()……………………………………….	16

    - AISS16AO2_66_Clear_Input_Buffer()………………………………………	17

    - AISS16AO2_66_Enable_Input_Buffer()………………………………….…	18

    - AISS16AO2_66_Disable_Input_Buffer()……………………………………	19

    - AISS16AO2_66_EnableInterrupt()…………………………………………..	20

    - AISS16AO2_66_DisableInterrupt()………………………………………….	21

    - AISS16AO2_66_Open_DMA_Channel()…………………………………….	22

    - AISS16AO2_66_DMA_FROM_Buffer()…..………………………………….	23

    - AISS16AO2_66_DMA_TO_Buffer()…..……..……………………………….	24

    - AISS16AO2_66_Close_DMA_Channel()……………………………………	25

    - AISS16AO2_66_Register_Interrupt_Notify()………………….……………	26

    - AISS16AO2_66_Cancel_Interrupt_Notify()………………………………..	27

    - AISS16AO2_66_Set_Output_Type()………………………………………..	28

    - AISS16AO2_66_Enable_Output_Clock ()..………………………………..	29

    - AISS16AO2_66_Disable_Output_Clock()…..……………………………..	30

    - AISS16AO2_66_Clear_Output_Buffer()……..……………………………..	31

    - AISS16AO2_66_Set_Input_Voltage_Range ()...…………………………..	32

    - AISS16AO2_66_Set_Input_Sample_Rate ()...……………………………..	33

    - AISS16AO2_66_Set_Output_Sample_Rate ()...…………………………..	34

    - AISS16AO2_66_Set_Output_Voltage_Range()….………………………..	35

- Driver Installation………………………………………………………………………...…	36

- Example Program………………………………………………………………………...…	37

- Scope

The Purpose of this document is to describe how to interface with the 16AISS16AO2 Windows Driver API developed by General Standards Corporation (GSC). This software provides the interface between the “Application Software” and the 16AISS16AO2 board.

The 16AISS16AO2 Driver API Software executes under control of the Windows Operating System. The 16AISS16AO2 is implemented as a standard Windows driver API written in “C” programming language. The 16AISS16AO2 Driver API Software is designed to operate on CPU boards containing x86 processors.

The 16AISS16AO2 Driver consists of a Windows driver with an interface layer (GSC API) to simplify the interface to the PLX Driver. While an application may interface directly to the PLX driver, interfacing to the GSC API layer, will simplify the application software development.

- Hardware Overview

The PMC-16AISS16AO2 board is a single-width PCI mezzanine card (PMC) that provides     16-bit analog input/output capability for PMC applications. 16 analog input lines are

configured as 16 differential channels, and are

digitized at rates up to 1,000,000 conversions per second per channel. The voltage range for analog inputs is software controlled as ±2.5V, ±5V or ±10V.

Two 16bit single-ended analog outputs provide software controlled ranges of  ±2.5V, ±5V or ±10V. A 16-bit bidirectional digital I/O port also is provided. The board is functionally compatible with the IEEE PCI local bus specification Revision 2.3. A PCI interface adapter supports the "plug-nplay" initialization concept.

A PCI interface adapter provides the interface between the controlling PCI bus and the internal local controller through a 16-bit local bus. The local controller manages all input/output configuration and data manipulation functions, including auto calibration.  Analog output levels are initialized to zero (midrange).  Multiboard synchronization is supported.

Selftest networks permit all channels to be calibrated automatically to a single internal voltage reference.  Offset and gain trimming of the output channels are performed by calibration DAC's that are loaded with channel correction values during initialization.  The correction values are determined during auto calibration, and are stored in nonvolatile EEprom for subsequent transfer to the calibration DAC's.  Either auto calibration or initialization can be invoked at any time by asserting a single control bit in the board control register.

The board is designed for minimum off-line maintenance, and includes internal monitoring features that eliminate the need for disconnecting or removing the module from the system for calibration.  All analog input and output system connections are made through a single 80-pin, dual-ribbon front-access  I/O connector. Power requirements consist of +5 VDC, in compliance with the PCI specification, and operation over the specified temperature range is achieved with conventional convection cooling.

- Referenced Documents

The following documents provide reference material for the 16AISS16AO2 board:

  - PMC-16AISS16AO2 User’s Manual – GSC

  - PLX Technology, Inc. PCI 9056 PCI Bus Master Interface Chip data sheet.

4.	General Standards API

This section describes the interface to the 16AISS16AO2 GSC API. The 16AISS16AO2 GSC API isolates the user from operating system specific requirements, allowing the API to be used with all Windows operating systems (98\NT\W2K\XP).

The 16AISS16AO2 Win Driver provides an interface to a 16AISS16AO2 card and a Windows application, which run on a x86 target processor. The driver is installed and devices are created when the driver is started during boot up. The functions of the driver can then be used to access the board. Devices are created with the name “board x” where “x” is the device number. Device numbers start at 1 and for each board found the device number will increment.

Included in the board driver software is a menu driven board application program. This program is delivered undocumented and unsupported but may be used to exercise the card and the device driver. It can also be used as an example for programming the 16AISS16AO2 device.

The user interfaces to the GSC API at the basic level with the following functions:

- Find Boards() - Detects all PLX Devices connected via the PCI Bus.

- Get Handle() - Opens a driver interface to one 16AISS16AO2 card.

- Readlocal32() - Reads local registers from one 16AISS16AO2 card.

- Writelocal32() - Writes to local Registers of one 16AISS16AO2 card.

- Close Handle() - Closes a driver interface to one 16AISS16AO2 card.

The user MUST call Find Boards to determine what PLX devices are installed in the system, and get the associated board number. The user then calls the Get Handle function with each board number to be used. This function obtains a handle to the device and initializes the device parameters within the API / driver. The user is then free (assuming no errors) to write / read the registers as desired. The user should always call Close Handle when done to free resources prior to exiting.

The function definitions and parameters are defined in the following paragraphs of this section.

  - AISS16AO2_66_FindBoards()

Detects all PLX Devices connected via the PCI Bus.

Prototype:

U32  AISS16AO2_66_FindBoards  	(char 		*pDeviceInfo,

U32		 *ulError);

Returns – Total number of PLX boards found in the system or –1L if error or no boards found.

Where:

pDeviceInfo – Contains “Board #: Bus: Slot: Type: Ser#”  info for PLX boards found.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

  - AISS16AO2_66_Get_Handle

Initializes Handle for the passed board number IN THE DRIVER.

Prototype:

U32  AISS16AO2_66_Get_Handle	(U32 		*ulError,

U32 		BoardNumber);

Returns – Error code if invalid board number passed (0, >31), else # boards.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

  - AISS16AO2_66_Read_Local32

Read a value from the board local register.

Prototype:

U32  AISS16AO2_66_Read_Local32	(U32 		BoardNumber,

U32 		*ulError,

U16 		iRegister);

Returns – Value read from the register.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

iRegister – Register to read. Values defined in 16AISS16AO2intface.h

BCR                 0x00

DIO_IO              0x04

AOUT_00             0x08

AOUT_01             0x0C

RESERVED            0x10

AIN_CHANS           0x14

AIN_DATA            0x18

RATE_A              0x1C

RATE_B              0x20

AIN_BURST_SIZE      0x24

AIN_BUFF_SIZE       0x28

AIN_BUFF_THRSHLD    0x2C

ICR                 0x30

BRD_CONFIG          0x34

AUTOCAL             0x38

BUFF_OUT_OPS        0x3C

AOUT_BUFF_THRSHLD   0x40

AOUT_BUFF_SIZE      0x44

AOUT_DATA           0x48

RATE_C	  	   0x4C

  - AISS16AO2_66_Write_Local32

Write a value to the board local register.

Prototype:

void  AISS16AO2_66_Write_Local32	(U32 		BoardNumber,

U32 		*ulError,

U16 		iRegister

U32		uiValue);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

iRegister – Register to write. Values defined in 16AISS16AO2intface.h

BCR                 0x00

DIO_IO              0x04

AOUT_00             0x08

AOUT_01             0x0C

RESERVED            0x10

AIN_CHANS           0x14

AIN_DATA            0x18

RATE_A              0x1C

RATE_B              0x20

AIN_BURST_SIZE      0x24

AIN_BUFF_SIZE       0x28

AIN_BUFF_THRSHLD    0x2C

ICR                 0x30

BRD_CONFIG          0x34

AUTOCAL             0x38

BUFF_OUT_OPS        0x3C

AOUT_BUFF_THRSHLD   0x40

AOUT_BUFF_SIZE      0x44

AOUT_DATA           0x48

RATE_C	  	   0x4C

uiValue – Value to write to the selected register.

Refer to the 16AISS16AO2 user manual for all register / bit definitions.

  - AISS16AO2_66_Close_Handle

Closes the device handle and frees the resources.

Prototype:

void  AISS16AO2_66_Close_Handle	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

  - Interface Functions

These functions allow the user to perform certain operations on the board, without having to keep track of individual register values and bit definitions.

    - AISS16AO2_66_Initialize

Perform a reset on the board. All register values are set to defaults. This Function does NOT wait for Initialization to complete, such that multiple boards can be initialized without waiting for each to finish.

Prototype:

void  AISS16AO2_66_Initialize	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Autocal

Perform an auto calibration on the board. This operation generates new calibration correction values which are stored in nonvolatile EEprom. This Function waits for Autocal to complete, and returns status.

Prototype:

U32  AISS16AO2_66_Autocal	(U32 		BoardNumber,

U32 		*ulError);

Returns – 0xAA for Interrupt timeout, or autocal status (0=FAIL or 1=PASS).

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Set_Input_Mode

Sets the input mode of the board: Differential, Single-Ended, Selftest (zero or Vref) or output channel (0-1) monitor.

Prototype:

void  AISS16AO2_66_Set_Input_Mode	(U32 		BoardNumber,

U32 		*ulError

U32		ulInputMode);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

ulInputMode – Valid values: 0 – 5

0 = Differential

1 = Single-Ended

2 = Zero Selftest

3 = +Vref Selftest

4 = Monitor Ch0 Output

5 = Monitor Ch1 Output

    - AISS16AO2_66_Set_Output_Mode

Sets the Output mode of the board: Immediate or Simultaneous.

Prototype:

void  AISS16AO2_66_Set_Output_Mode	(U32 		BoardNumber,

U32 		*ulError

U32		ulOutputMode);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

ulOutputMode – Valid values: 0 = Immediate, 1 = Simultaneous

    - AISS16AO2_66_Clear_Input_Buffer

Clears all data from the input buffer.

Prototype:

void  AISS16AO2_66_Clear_Input_Buffer	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Enable_Input_Buffer

Enables data collection to the input buffer.

Prototype:

void  AISS16AO2_66_Enable_Input_Buffer	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Disable_Input_Buffer

Disables data collection to the input buffer.

Prototype:

void  AISS16AO2_66_Disable_Input_Buffer	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_EnableInterrupt

Enables the desired interrupt in the local register, and for the PCI bus. See 16AISS16AO2 User manual for interrupt sources.

Prototype:

U32  AISS16AO2_66_EnableInterrupt	(U32 		BoardNumber,

U32		ulValue,

U32		ulType,

U32 		*ulError);

Returns – Interrupt value set.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulValue – The desired interrupt value to set, valid for 0 – 0xFFFF.

ulType – Valid for 0 (LOCAL) or 1 (DMA).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_DisableInterrupt

Disables all interrupts in the local register, and for the PCI bus.

Prototype:

void  AISS16AO2_66_DisableInterrupt	(U32 		BoardNumber,

U32		ulValue,

U32		ulType,

U32 		*ulError);

Returns – N/A.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulValue – The desired interrupt value to clear, valid for 0 – 0xFFFF.

ulType – Valid for 0 (LOCAL) or 1 (DMA).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Open_DMA_Channel

Opens the desired DMA channel for transferring data from the input FIFO, or to the output FIFO.

Prototype:

void  AISS16AO2_66_Open_DMA_Channel(U32 		BoardNumber,

U32		ulChannel,

U32		ulMode,

U32 		*ulError);

Returns – N/A.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulChannel – The desired channel to open, valid for channel 0 or 1.

ulMode – Valid for 0 (non Demand Mode) or 1 (Demand Mode).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_DMA_FROM_Buffer

Transfers the desired number of WORDS from the board input buffer.

Prototype:

U32  AISS16AO2_66_DMA_FROM_Buffer  (U32 		BoardNumber,

U32		ulChannel,

U32		ulWords,

U32*		uData,

U32 		*ulError);

Returns – WORDS transferred if no error.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulChannel – The DMA channel previously opened, valid for channel 0 or 1.

ulWords – Number of WORDS to transfer. (BYTES = ulWords*4).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_DMA_TO_Buffer

Transfers the desired number of WORDS to the board output buffer.

Prototype:

U32  AISS16AO2_66_DMA_TO_Buffer  (U32 		BoardNumber,

U32		ulChannel,

U32		ulWords,

U32*		uData,

U32 		*ulError);

Returns – WORDS transferred if no error.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulChannel – The DMA channel previously opened, valid for channel 0 or 1.

ulWords – Number of WORDS to transfer. (BYTES = ulWords*4).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Close_DMA_Channel

Closes the desired DMA channel.

Prototype:

void  AISS16AO2_66_Close_DMA_Channel(U32 	BoardNumber,

U32		ulChannel,

U32 	*ulError);

Returns – N/A.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

uiValue – The desired channel to close, valid for channel 0 or 1.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Register_Interrupt_Notify

Registers a user event for interrupt notification.

NOTE: This function DOES NOT enable the interrupts the notification is based on.

Prototype:

void  AISS16AO2_66_Register_Interrupt_Notify	(U32 		BoardNumber,

GS_NOTIFY_OBJECT 		*event,

U32 		ulIntr,

U32 		ulType,

U32 		*ulError);

Returns – N/A.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

event – User creates an event and stores the handle in a GS_NOTIFY_OBJECT structure member hEvent. The user should not set or change the other members of the structure.

ulIntr – When ulType equals one (1).  Valid for 0 (DMA0) or 1 (DMA1).

When ulType equals zero(0). See manual for valid values / sources.

ulType – Determines interrupt type to attach to. Valid for 0 (Local) or 1 (DMA).

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Cancel_Interrupt_Notify

Cancels interrupt notification for a user event.

Prototype:

void  AISS16AO2_66_Cancel_Interrupt_Notify	(U32 		BoardNumber,

GS_NOTIFY_OBJECT 		*event,

U32 		*ulError);

Returns – N/A.

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

event – A valid GS_NOTIFY_OBJECT structure which has been initialized by a call to AISS16AO2_66_Register_Interrupt_Notify.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Set_Output_Type

Sets the Output type of the board: Register or FIFO.

Prototype:

void  AISS16AO2_66_Set_Output_Type	(U32 		BoardNumber,

U32 		*ulError,

U32		ulOutputType);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

ulOutputType – Valid values: 0 = Register, 1 = FIFO

    - AISS16AO2_66_Enable_Output_Clock

Enables the analog output clock of the board.

Prototype:

void  AISS16AO2_66_Enable_Output_Clock(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Disable_Output_Clock

Disables the analog output clock of the board.

Prototype:

void  AISS16AO2_66_Disable_Output_Clock(U32 	BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Clear_Output_Buffer

Empties the analog output FIFO.

Prototype:

void  AISS16AO2_66_Clear_Output_Buffer	(U32 		BoardNumber,

U32 		*ulError);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Set_Input_Voltage_Range

Sets the analog input voltage of the board for the two groups.

Prototype:

void  AISS16AO2_66_Set_Input_Voltage_Range	(U32 		BoardNumber,

U32 		*ulError,

U32		ulRangeA_B,

Double	vRange);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

ulRangeA_B – Valid values: 0 = Chans 0,1,4,5,8,9,12,13: 1 = Chans 2,3,6,7,10,11,14,15

vRange – Valid values: 2.50, 5.00, or 10.0

    - AISS16AO2_66_Set_Input_Sample_Rate

Sets the input sample rate for all input channels of the board. The function sets RATE_A for continuous mode, or RATE_B for burst mode. The enable burst input bit must be set in the BCR register prior to calling this function to set RATE_B.

NOTE: fClock is currently 45Mhz.

Prototype:

double AISS16AO2_66_Set_Input_Sample_Rate	(U32 		BoardNumber,

Double	fRate,

U32 		*ulError);

Returns – Actual sample rate (fClock / nDiv).

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

fRate – Valid values: 1KHz – 1Mhz.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Set_Output_Sample_Rate

Sets the output sample rate for all output channels of the board. The function sets the RATE_C register.

NOTE: fClock is currently 45Mhz.

Prototype:

double AISS16AO2_66_Set_Output_Sample_Rate	(U32 		BoardNumber,

Double	fRate,

U32 		*ulError);

Returns – Actual sample rate (fClock / nDiv).

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

fRate – Valid values: 3Hz – 1Mhz.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

    - AISS16AO2_66_Set_Output_Voltage_Range

Sets the Output voltage range for all channels of the board.

Prototype:

void  AISS16AO2_66_Set_Output_Voltage_Range	(U32 		BoardNumber,

U32 		*ulError,

Double	vRange);

Returns – N/A

Where:

BoardNumber – Defines board number to be used by the driver for a particular device.

ulError – Returns 0 or error code. Refer to tools.h for a list of error codes.

vRange – Valid values: 2.50, 5.00, or 10.0.

5 Driver Installation

This section details driver installation on the target system. Any current driver previously installed for the 16AISS16AO2 must be uninstalled prior to this installation to avoid interference.

To install the driver, API, and associated example files, insert the CD ROM into the drive and close the bay. The installation should commence automatically and display user prompts. Follow the onscreen instructions to complete the installation.

Should the installation fail to automatically start, Select Start  Run  Browse on the Windows toolbar/popup and browse to find Setup.exe on the CD ROM. Click on OK to commence the installation.

The following files are installed on the target system:

OS dependent\…\GS66AISS16.sys [32bit Only]

OS dependent\…\GS66AISS16M.sys [64bit Only]

OS dependent\…\GSebApi.dll

Program Files\General Standards\66AISS16AO2\Example.exe

Program Files\General Standards\66AISS16AO2\AISS16AO2 eDriver C.dll

Program Files\General Standards\66AISS16AO2\AISS16AO2 eDriver C.lib

Program Files\General Standards\66AISS16AO2\16AISS16AO2eintface.h

Program Files\General Standards\66AISS16AO2\16AISS16AO2_eExample.c

Program Files\General Standards\66AISS16AO2\Tools.c

Program Files\General Standards\66AISS16AO2\Tools.h

Program Files\General Standards\66AISS16AO2\CioColor.h

OS Dependent

Program Files\General Standards\66AISS16AO2\66-AISS16ebdriver.inf

Program Files\General Standards\66AISS16AO2\66-AISS16AO2driver.inf

6.	Example Program

This section describes the example program, and the files required to develop an application.

The complied example program allows the user to exercise the installed device, while observing the outputs. To execute, double click on ‘Example.exe’. Refer to the Driver Installation section for file location.

The source is provided to educate the user with the GSC API function calls and provide a working example to aid the user with application development. To build the example program using MS Visual C++, create a project and add the following files:

Source Files 	 16AISS16AO2_eExample.c

 Tools.c

Header Files 	 16AISS16AO2eintface.h

 CioColor.h

 Tools.h

Resource Files 	 AISS16AO2 eDriver C.lib

Select Build  [ProjectName].exe on the toolbar.

NOTE: AISS16AO2 eDriver C.dll must be in the project directory to run the example.

Contact GSC for example programs (drivers) for other development environments (i.e LabVIEW, LabWindows/CVI, etc.)